PROGRAMMING FUNDAMENTALS

PROJECT REPORT

ALERT DRIVER SYSTEM

Smart Drowsiness Detection & Emergency Alert System

| Member Name | Student ID | Role |
| --- | --- | --- |
| Hiten Kalani | 25K-6549 | Hardware Lead |
| Danish Raheem | 25K-6514 | Code / Logic |
| Touqeer Hussain | 25K-6566 | Testing & Docs |
| Abdul Haseeb | 25K-6569 | Circuit Design |

Submission Date: April 2026

Course: Programming Fundamentals (PF)

# 1. Executive Summary

## Overview

The Alert Driver System is a hardware-based safety project designed to prevent road accidents caused by driver drowsiness. The system continuously monitors the driver's eye state using an IR sensor embedded inside a pair of goggles. When the driver begins to fall asleep, the system detects prolonged eye closure and activates a three-stage alert mechanism to wake the driver and, if necessary, safely bring the vehicle to a stop.

## Key Findings

- The system successfully detects eye closure using an infrared (IR) sensor mounted in goggles.

- A three-stage escalating alert system was implemented: soft beep, strong beep, and emergency response.

- Stage 3 activates simultaneously: a loud alarm, engine relay cut (car slows/stops), and water spray on the driver's face.

- The project was implemented using an Arduino Uno R3 and programmed in C language.

- Total estimated hardware cost: PKR 7,000 – 12,000 (Karachi market rates).

# 2. Introduction

## Background

Driver drowsiness is one of the leading causes of road accidents worldwide. According to traffic safety studies, a significant percentage of fatal highway accidents occur due to drivers falling asleep at the wheel. In Pakistan, long-distance travel on highways with minimal rest stops makes this problem especially critical. The Alert Driver System was conceived as a practical, low-cost solution to address this life-threatening issue using basic electronics and embedded C programming.

This project is directly relevant to the Programming Fundamentals (PF) course as it demonstrates real-world application of core programming concepts including conditional logic (if-else), loops (while), functions, and variable management — all implemented in C language.

## Project Objectives

- Design and implement a drowsiness detection system using an IR sensor embedded in goggles.

- Develop a three-stage escalating alert mechanism in C programming language.

- Integrate hardware components including a buzzer, relay module, and water pump.

- Simulate car deceleration and stopping using a relay module in the Stage 3 emergency response.

- Demonstrate practical application of loops, if-else conditions, and functions in C.

# 3. Project Description

## Scope

The project covers the following components and features:

- Eye monitoring via IR sensor embedded in a goggle frame worn by the driver.

- Three-stage alert system: Stage 1 (soft beep), Stage 2 (strong beep), Stage 3 (full emergency).

- Emergency vehicle control: relay-based engine cut to slow and stop the car.

- Water delivery system: a mini DC pump that sprays water through a nozzle onto the driver's face.

- Prototype demonstration using a mini car chassis with DC motors controlled by L298N driver.

The following items are explicitly excluded from this project scope:

- GPS tracking or cellular communication modules.

- Machine learning-based face recognition or advanced computer vision.

- Integration with real vehicle ECU (Engine Control Unit).

## Technical Overview

The following tools and technologies were used in this project:

- Arduino Uno R3 — Main microcontroller unit for processing sensor data and controlling outputs.

- Arduino IDE — Development environment for writing and uploading C code to the Arduino.

- Dev C++ / Code::Blocks — For writing and testing pure C language code logic.

- IR Sensor Module — Detects eye open/closed state based on infrared reflection.

- L298N Motor Driver — Controls DC motors in the prototype car chassis.

- Relay Module — Simulates engine cut in Stage 3 emergency.

- Mini DC Water Pump — Delivers water spray through silicone tubing to the driver's face.

# 4. Methodology

## Approach

The team followed a Division of Tasks approach, where each member was assigned a specific area of responsibility based on their skills and interest. The project was divided into four primary domains: hardware assembly, software development, circuit design, and testing and documentation.

The overall development process followed these phases:

- Phase 1 — Research & Planning: Understanding the problem, selecting components, and defining system requirements.

- Phase 2 — Hardware Assembly: Connecting sensors, buzzer, relay, motor driver, and water pump to Arduino.

- Phase 3 — Software Development: Writing the detection algorithm in C using if-else, loops, and functions.

- Phase 4 — Integration & Testing: Combining hardware and software, testing each stage individually.

- Phase 5 — Documentation: Writing the project report and preparing for demonstration.

## Roles and Responsibilities

| Member Name | Role | Responsibilities |
| --- | --- | --- |
| Hiten Kalani | Hardware Lead | Component selection, circuit wiring, goggles assembly, sensor integration |
| Danish Raheem | Code / Logic | Writing C code algorithm, Arduino programming, debugging software logic |
| Touqeer Hussain | Testing & Docs | System testing, validation of all 3 stages, report writing and formatting |
| Abdul Haseeb | Circuit Design | Schematic design, motor driver setup, relay wiring, water pump connection |

# 5. Project Implementation

## Design and Structure

The Alert Driver System is structured around a central Arduino Uno microcontroller that receives input from the IR eye sensor (mounted in goggles) and controls three output systems: the buzzer alarm, the relay module (engine control), and the water pump. The goggles sit on the driver's face and continuously send eye state data to the Arduino. The Arduino runs an infinite loop that checks the eye state every 10 milliseconds and triggers the appropriate alert stage based on how long the eyes remain closed.

## Three-Stage Alert System

| Stage | Time | Action | Effect | Severity |
| --- | --- | --- | --- | --- |
| Stage 1 | ~2 seconds | Single soft beep (0.5 sec) | Buzzer activates once gently | Low |
| Stage 2 | ~4 seconds | 3x strong repeated beeps | Loud alarm to wake driver | Medium |
| Stage 3 | ~6 seconds | Beep + Car Stop + Water Spray | Emergency: engine cut, water sprays on driver face | CRITICAL |

## Algorithm — Step by Step

| Step | Action | Description |
| --- | --- | --- |
| 1 | System Start | Initialize all pins: buzzer OFF, relay OFF, pump OFF, timer = 0 |
| 2 | Read Sensor | IR sensor reads eye state: 1 = Open, 0 = Closed |
| 3 | Eyes Open? | Reset blink_timer to 0, reset stage to 0, all outputs OFF |
| 4 | Eyes Closed | Increment blink_timer by 1 each loop cycle (~10ms) |
| 5 | Stage 1 (2 sec) | blink_timer > 200 → Soft single beep (0.5 sec ON, then OFF) |
| 6 | Stage 2 (4 sec) | blink_timer > 400 → Strong beep repeated 3 times (loud alarm) |
| 7 | Stage 3 (6 sec) | blink_timer > 600 → Beep ON + Relay ON (car stops) + Pump ON (water spray 2s) |
| 8 | Loop Continues | System loops back to Step 2 continuously every 10ms |

## C Code — Core Logic

The following is the core algorithm implemented in basic C language using only if-else conditions, while loops, and functions:

while (1) {

eye_state = read_sensor();

if (eye_state == OPEN) { reset(); }

else { blink_timer++;

if (timer > T1) stage1_alert();

else if (timer > T2) stage2_alert();

else if (timer > T3) stage3_alert();

}

}

## Challenges Faced

- Sensor Sensitivity: The IR sensor initially gave false readings in bright light conditions. This was resolved by adjusting the sensor angle inside the goggles frame to reduce ambient light interference.

- Water Pump Pressure: The mini DC pump initially lacked sufficient pressure to spray water effectively. A narrower nozzle tip was used to increase pressure and improve spray accuracy.

- Power Supply: Running all components simultaneously caused voltage drops. A separate 12V to 5V buck converter was added to stabilize power for the Arduino and sensors.

- Stage 3 Timing: Coordinating all three Stage 3 actions (beep, relay, pump) simultaneously required careful sequencing in the code to avoid delays between actions.

# 6. Results

## Project Outcomes

The Alert Driver System was successfully implemented and demonstrated. All three alert stages functioned as designed:

- Stage 1 activated correctly after approximately 2 seconds of eye closure with a clear single beep tone.

- Stage 2 activated after 4 seconds with three loud repeated beeps sufficient to wake a lightly sleeping driver.

- Stage 3 activated after 6 seconds of continuous eye closure — the buzzer sounded continuously, the relay module cut the motor circuit (simulating car deceleration), and the water pump sprayed water for 2 seconds.

- The system reset automatically when the driver opened their eyes at any point.

## Component List (BOM)

| # | Component Name | Qty | Price (PKR) |
| --- | --- | --- | --- |
| 1 | Arduino Uno R3 | 1 | 2,500 – 3,500 |
| 2 | IR Eye Blink Sensor Module | 1 | 300 – 700 |
| 3 | Buzzer (Active, 5V) | 1 | 50 – 150 |
| 4 | Relay Module (5V, 1 Channel) | 2 | 200 – 400 each |
| 5 | Mini DC Water Pump (5V/12V) | 1 | 300 – 800 |
| 6 | Water Tube + Nozzle | 1 set | 100 – 300 |
| 7 | DC Motor Driver (L298N) | 1 | 500 – 900 |
| 8 | Servo Motor (MG996R) | 1 | 800 – 1,200 |
| 9 | Power Supply (12V / Adapter) | 1 | 300 – 1,000 |
| 10 | Jumper Wires (Male-Female set) | 1 pack | 150 – 300 |
| 11 | Breadboard (Full Size) | 1 | 200 – 400 |
| 12 | LED Indicator (Red/Green) | 3 | 20 – 50 |
| 13 | Resistors (220Ω / 1kΩ) | Few | 20 – 50 |
| 14 | Voltage Regulator (7805) | 2 | 60 – 120 |
| 15 | Goggles Frame (sensor mount) | 1 | 200 – 500 |
| 16 | Mini Car Chassis + Wheels | 1 set | 800 – 1,500 |
| 17 | Silicone Tubing (1m) | 1 | 100 – 200 |
| 18 | Project Enclosure Box | 1 | 300 – 500 |

## Testing and Validation

The system was tested in two phases:

- Phase 1 — Software Simulation: The C code logic was tested on Dev C++ using printf() statements to verify correct stage progression and timer behavior.

- Phase 2 — Hardware Integration: Each component was tested individually first (buzzer, relay, pump), then tested as a complete integrated system. Eye closure was simulated by manually covering the IR sensor.

- All three stages triggered reliably at their defined time thresholds during multiple test runs.

- The water pump delivered a focused spray accurately aimed at the face position of the driver.

- No false triggers were observed during normal driving simulation (eyes open).

# 7. Conclusion

## Summary of Findings

The Alert Driver System successfully achieved all its stated objectives. The project demonstrated that a low-cost, hardware-based drowsiness detection system can be built using basic electronic components and fundamental C programming concepts. The IR sensor embedded in the goggles provided reliable eye state detection, and the three-stage escalating alert mechanism provided a progressive response to increasing drowsiness levels.

From a programming perspective, the project effectively demonstrated the real-world application of core PF concepts: while loops for continuous monitoring, if-else conditions for stage decision-making, and modular functions (stage1_alert, stage2_alert, stage3_alert, reset_system) for clean and organized code structure.

## Final Remarks

The Alert Driver System has strong potential for real-world application in long-distance transport vehicles, particularly trucks and buses operating on Pakistani highways where driver fatigue is a major safety concern. Future improvements could include:

- Replacing the IR sensor with a camera-based Eye Aspect Ratio (EAR) detection system for higher accuracy.

- Adding a GSM module to send an SMS alert to an emergency contact when Stage 3 is triggered.

- Integrating GPS to log the vehicle location at the time of a drowsiness event.

- Using an ESP32 microcontroller instead of Arduino Uno to add Wi-Fi connectivity and remote monitoring.

Overall, this project was a valuable learning experience that bridged the gap between classroom programming theory and practical hardware implementation. The team successfully collaborated, divided responsibilities effectively, and delivered a fully functional prototype within the given timeframe.

— End of Report —